Electrical network analysis system for quality control of your facilities

The device measures all the electrical parameters in terms of energy quality according to standards IEC 61000-4-30 Class A and IEC 62586-2 Class A : harmonic numbers, dips /cut-offs, overvoltages, transients, wave form capture and verification of signal conformity according to the templates defined in standard EN 50160.

The qualimeter has the following main features:

- 4 voltage and 5 current channels

- Voltage inputs for direct connection until 1000 Vac phase-phase or through Voltage Transformer

- Current inputs availability for standard CT 1A/5A

- Voltage characteristics monitoring according to  IEC/EN 61000-4-30:2015 Ed. 3 Class A

- Parameter recording for statistical analysis according to EN 50160

- Harmonic and interharmonics measurement

- Voltage Flicker measurement

- Main Signalling Voltage detection and recording

- EN50160 Event detection and recording (sag, swell, outage)

- Current Overload detection and recording

- Transient detection and recording ( down to 20 microseconds)

- High sampling frequency of 50 kHz

- Three-phase 3 and 4 wire or single-phase operating mode

- Integrated web server for complete device management

- Two Ethernet and one RS485 ports for easy communication

- One front USB host port for memory stick

- RTC synchronization for accurate time information by NTP or internal GPS module

- 16 Gb internal memory for data recording

- Data recorded in standard PQDIF and CSV formats

- Panel mount 192x144 DIN size

- High resolution TFT screen with capacitive touch

- 4x Isolated digital inputs, 4x digital outputs and 4x 4-20mA analog outputs

- Integrated backup rechargeable battery

- Multi measurement of currents, voltages, powers, power factors, harmonics and energies

- Communication protocols: HTTP, HTTPS, FTP, SFTP, NTP, Modbus TCP

This device made for power quality analysis will able to detect and record events such as:

- Voltage events recording (sags, swells, interruptions) with storage of 1/2c RMS values and waveforms

- Rapid voltage changes characteristics

- Fast current variation recording with storage of RMS values and waveforms

- Data LOG recording function of MIN/AVG/MAX values of selectable parameters and programmable rate time

- Functional event LOG for the device operating status

- Digital input status change LOG

- Automatic data upload to FTP server in case of events

- Scheduled data upload to FTP server

The qualimeter conforms to the following standards and certifications:

- Active and apparent energy: IEC 62053-22 class 0.5s

- Reactive energy: IEC 62053-24 class 1

- Harmonics and interharmonics measurement method: IEC 61000-4-7

- Flicker measurement method: IEC 61000-4-15 and EN 6100-4-15

- Supply quality measurement method: IEC 61000-4-30 Class A

- Power quality measurement requirements: IEC 62586-2 Class A

- Characteristics of the voltage supplied by public distribution networks: EN50160.

The product is similar to Socomec Diris Q800 network analyzer.